# SpeedTruth, Investment Corridor Short-list

From flagged segments to fundable corridors. A development bank invests in corridors, not isolated 150 m segments, so SpeedTruth groups the flagged Priority-1/2 segments into contiguous, same-road investment corridors and ranks them by where avoidable harm concentrates. ADB AI for Safer Roads 2026 · Thailand & Maharashtra (India) · derived outputs only (FDUA-safe)

## The headline for an investment committee

- 3,679 flagged segments collapse into 2,869 corridors (405 multi-segment routes; the rest single-segment spots).

- The top 20% of corridors carry ~76% of the total avoidable-harm benefit. Funding flows to a short-list, not a 3,679-line spreadsheet.

- Each corridor carries a dominant intervention class, a cost tier, an iRAP-style benefit-cost index, and a coordinate to inspect on the ground.

## Method (transparent, reproducible, src/corridors.py)

Two flagged segments join the same corridor if they are spatially contiguous (within 60 m, metric UTM) and share a road name; connected components of that graph are corridors. Corridors are ranked by total avoidable-harm benefit = sum over segments of (Power-Model KSI-reduction x exposure), the same proxy used in the investment case (FINDINGS.md §4). This is a transparent, relative prioritisation signal, not a casualty forecast (no crash data; see FINDINGS.md §6).

## Top 15 corridors by avoidable-harm benefit

| Rank | Corridor (road) | Region | Segs | Length (km) | Critical segs | Dominant intervention | BCR index | Inspect at |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Phahonyothin Road | THA | 4 | 175.3 | 4 | Footways / crossings / separation | ~24 | 17.23462,99.13542 |
| 2 | Si Khio-Det Udom Road | THA | 10 | 179.2 | 10 | Footways / crossings / separation | ~9 | 14.61997,103.28428 |
| 3 | Phetkasem Road | THA | 5 | 163.5 | 5 | Footways / crossings / separation | ~18 | 10.99824,99.32187 |
| 4 | Phahon Yothin Road | THA | 5 | 168.7 | 5 | Footways / crossings / separation | ~18 | 14.13177,100.64946 |
| 5 | Suwannason Road | THA | 8 | 167.5 | 8 | Footways / crossings / separation | ~11 | 13.81547,102.16090 |
| 6 | Debaratna Road | THA | 4 | 163.9 | 4 | Footways / crossings / separation | ~21 | 13.60145,100.80185 |
| 7 | Phahon Yothin Road | THA | 11 | 160.2 | 11 | Footways / crossings / separation | ~8 | 16.00510,99.82070 |
| 8 | Mittraphap Road | THA | 6 | 161.2 | 6 | Footways / crossings / separation | ~14 | 14.92241,101.90390 |
| 9 | Nittayo Road | THA | 4 | 159.7 | 4 | Footways / crossings / separation | ~21 | 17.31559,104.42867 |
| 10 | Rama II Road | THA | 4 | 155.1 | 4 | Footways / crossings / separation | ~20 | 13.59204,100.32433 |
| 11 | Maliwan Road | THA | 11 | 158.8 | 11 | Footways / crossings / separation | ~7 | 16.49846,102.40268 |
| 12 | (unnamed corridor) | THA | 1 | 152.2 | 1 | Footways / crossings / separation | ~80 | 16.95463,103.55928 |
| 13 | Sukhumvit Road | THA | 6 | 147.2 | 6 | Footways / crossings / separation | ~13 | 12.73480,101.74502 |
| 14 | Rangsit-Nakhon Nayok Road | THA | 4 | 142.0 | 4 | Footways / crossings / separation | ~19 | 14.10239,100.90959 |
| 15 | Asian Highway | THA | 4 | 134.3 | 4 | Footways / crossings / separation | ~18 | 7.91071,99.92769 |

Full ranked list: output/corridors_summary.csv (2,869 corridors). Interactive map: output/corridor_map.html. Geometry: output/corridors.geojson.

## How a ministry / ADB uses this

- Fund the short-list. Start at rank 1 and work down until the budget is exhausted; the benefit concentration means a few corridors capture most of the avoidable harm.

- Match the instrument to the corridor. Green corridors are near-zero-cost limit re-sets (immediate quick wins); red corridors need physical protection (footways, crossings, separated motorcycle lanes) and a capital line.

- Inspect before committing. Every corridor has a coordinate; open it in street imagery (the same StreetImageLink the model reads) to confirm before funding.

- Worked example: PILOT_CORRIDOR.md takes one corridor end-to-end, from flag to costed action.